������Ŀ ������Ŀ �����Ŀ  �����Ŀ �����Ŀ �����Ŀ �Ŀ �Ŀ �����Ŀ �����Ŀ

�������� �������� �������  ������� ������� ������� ��� ��� ������� �������

������Ŀ ������Ŀ ������Ŀ �����Ŀ ��Ŀ    �Ŀ     �����Ŀ �����Ŀ   �Ŀ

����Ŀ � � ��Ŀ � � ��Ŀ � � ڿ �� � ��    � �     � �Ŀ � � �Ŀ �   � �

������ � � �  � � � ���� � � �� �  � ���Ŀ � ���Ŀ � � � � � � � �   � �

�������� ���  ��� �������� ������  ������� ������� ��� ��� ��� ���   ���

SabreChat! Version 5.03.04 Copyright (C) 1993 Shawn Gresham

What is SabreChat!?

�������������������

SabreChat! is simply a user to user chat program. It is run

from each node of a bulletin board system allowing the users

online to talk with each other.  This connection is made

possible by the SabreMixer! TSR that comes with the SabreChat!

package.

This SabreChat! version has the following features:

Line-by-line or character-by-character chat modes that

are toggled by the user.

Communication between nodes separated by a multi-tasker or a

network in character-by-character mode.  Disk-based

communication in line-by-line mode.

Full ANSI color coding support.

Multiple conferencing support for more than one chat session

at the same time.

Supports up to 50 nodes.

Sysop-defined action commands for user entertainment.

Sysop-defined questionnaire feature to give each user

more identity.

Three different prompting modes for user identification.

Ability to capture conversations to a text file.

Log file generation.

By The Way

����������

SabreChat! is distributed as shareware with nothing stating

the user has to register the program within a certain number

of days.  The unregistered version of SabreChat! has a

built-in limitation of 10 minutes for each execution and no

other limitations.  There is no restriction for use by

corporations or govermental agencies.  The software is

distributed as is.  I am not liable for any damage cause by

the use of this software.

Trademarks of any companies mentioned in this doc appears for

identification purposes only and likewise, belong to their

respective companies.

Requirements

������������

The following are required to execute SabreChat!:

8086 or better

DOS 3.3 or newer

a communication fossil for port access

SHARE.EXE must be loaded

SabreChat! uses the following files:

File            Description                     Found

��������������� �����������������������         �������

SCHAT!.EXE      Main Executable                 archive

SCHT!WC.DOC     Documentation                   archive

SMIXER!.COM     SabreMixer!                     archive

SMIXER!.DOC     Doc for SabreMixer!             archive

SCHT!HI.ASC     welcome text (ASCII)            archive

SCHT!HI.ANS     welcome text (ANSI)             archive

SCHT!HLP.ASC    help text (ASCII)               archive

SCHT!HLP.ANS    help text (ANSI)                archive

SCHT!MNU.ASC    menu text (ASCII)               archive

SCHT!MNU.ANS    menu text (ANSI)                archive

SCHT!ACT.EXE    action compiler                 archive

SCHT!ACT.TXT    sample actions (text)           archive

SCHT!ACT.DAT    sample actions (compiled)       archive

SCHT!REG.EXE    registration program            archive

SCHT!KEY.DAT    registration key file           created

SCHT!QST.EXE    questionnaire compiler          archive

SCHT!ASK.TXT    questionnaire questions         archive

SCHT!QST.TXT    uncompiled questionnaire data   archive

SCHT!QST.DAT    compiled questionnaire data     created

SCHT!USR.SWD    user data                       created

SCHT!CNF.SWD    conference data                 created

SCHT!LIN.SWD    line data                       created

SCHT!LOG.SWD    log file                        created

SCHT!CAP.xxx    capture file for room xxx       created

SCHT!ALS.SWD    user alias information          created

SCHT!NFO.SWD    user questionnaire information  created

ARCHIVE = found in the SabreChat! archive.

CREATED = created by SabreChat!.

Registering SabreChat!

����������������������

The copy of SabreChat! that you possess is the full-working

version with only one drawback.  Users are limited to ten

minutes in the door before they are returned to the board.

Afterwards they may return to SabreChat! for another ten

minutes and so on.  This limitation is merely to allow the

user and sysop to have a sufficent evaluation periods to

determine if they wish to register SabreChat!.

Why register at all then?  Well, there are few reasons you

might consider...

1) Upon registration, you will be added to a list of names

that I personally inform when a newer version of SabreChat!

comes out.

2) You will receive priority support, in the unlikely event that

you run into any problems or if you just have some questions.

3) Registrations keep me informed on just how interested

people are in my software.  If I don't receive any feedback

then I can only assume that my efforts were in vain and I

should find something else to do.

4) Registrations are forever.  Once registered, you will never

have to register for a newer version of SabreChat!.

5) Upon receiving the registration key, the ten minute

limitation will be removed.

To register SabreChat!, please fill out the registration form

included in the archive and send fifteen US dollars ($15 US)

address found at the end of this document.

Be sure that the names given are the names that you wish to

appear in the registration.  The registration will look

similar to this:

Registered to SHAWN GRESHAM of MAXIMUM INTENSITY.

-------------    -----------------

The information is merely for my records so that I can keep

tabs on where the program is headed and such.  Plus the

comments and suggestion are invaluable to the production of

future versions.

Installation

������������

To install SabreChat! please read the installation text file

also included in this archive.  This fill will be called

INST!xxx.DOC with xxx being the bbs software type.

RA  = Remote Access

WC  = Wildcat

TAG = TAG BBS

Also note that though all of the installation docs for each

system (RA/WC/TAG) are included, only the system supported

by Schat!.exe will work.

NOTE:  I have included a user tutorial text file,

SCHT!USR.DOC, that can be set up as a bulletin on your

system to help people get the hang of SabreChat!.

Customizing SabreChat!

����������������������

Using the installation instructions, you should have a

standard working version of SabreChat!.  The following section

gives a list of command line options that you can use to

customize the way SabreChat! works on your system.  I would

recommend make very few changes until you are certain that the

base setup will operate correctly.   Customizing would

complicate debugging a problem.

Note that some options are already set by the DOOR.SYS or

DORINFO1.DEF file for you.  These commands are identified by

having [DOOR.SYS] or [DORINFO1.DEF] after their name. Normally

you would not have to set these commands unless you wanted to

over-ride the DOOR.SYS or DORINFO1.DEF settings.

The command line parameters are listed in the following

format:

Option <letter>, <short description> [DOOR.SYS] [DORINFO1.DEF]

<full description>

Usage   : SCHAT! <option usage description>

Default : SCHAT! <example>

Example of how to activate option from

command line.

DOOR.SYS

Means that the option is read from

DOOR.SYS if the Y option has been set.

DORINFO1.DEF

Means that this option will be set if

you use the O (read dorinfo1.def)

option.

None

Means that the option has no default

value or is not activated by default.

AutoDetect

Means that SabreChat! tries to

determine this value when it is

executed, and that by setting this

option, you are overriding the

detected value.

Option A, Select Graphics Mode [DOOR.SYS] [DORINFO1.DEF]

This command is set by DOOR.SYS and DORINFO1.DEF.

This command selects the graphic codes that SabreChat!

uses to send colors to an online user.  SabreChat!

supports two standard graphic conventions, ANSI and

AVATAR. This option will be set by DOOR.SYS and

DORINFO1.DEF, if you are using option Y or O, based on

the user's flags in RA.

Usage : SCHAT! A<0..3>

0       No graphic codes

1       ANSI graphics

2       AVATAR graphics

3       Both ANSI and AVATAR

Example : SCHAT! A1

Default : SCHAT! A0 or DOOR.SYS or DORINFO1.DEF

Option B, Black and White Local Screen

This command informs SabreChat! not to use colors

locally.   Color codes will still be sent to the user

online.  This is necessary on some black and white

monitors that translate certain colors as black and

cannot been seen against the black background.

Usage : SCHAT! B

Default : (color used)

Option C, Set Default Conference Name

This option sets the conference name that user will

be joined to when he enters SabreChat!.

Usage   : SCHAT! C<conference name>

Example : SCHAT! CCOMICS

Default : SCHAT! CPUBLIC

Option E, Enable DOS 3.3 COMMIT flushing

This option enables file flushing for some of the

temporary files that SabreChat! uses to communicate.

Enabling this will slow down operation a tad, but will

ensure that the data gets written to the disk as soon

as possible.  Do not use this unless some nodes are

receiving the wrong text lines (ie. old lines of text

or massive garbage, etc).

Usage   : SCHAT! E

Default : (inactive)

Option F, Set Page Search Frequency

This command defines the frequency that SabreChat!

checks to see if someone has paged you while in

SabreChat!. SabreChat! looks for pages once every 5

seconds by default.  Using the F command line option,

you can change this value or disable page checking

entirely by setting it to zero.

Usage   : SCHAT! F<frequency in seconds or 0 (disabled)>

Default : SCHAT! F5

Option G, Log Activity

This option creates a file called SCHT!LOG.SWD and

logs the doors activity.  You can safely delete this

file whenever it becomes to larger and SabreChat! will

create a new one.  This keeps a list of who enters and

leaves and any errors that may occur.

Usage: SCHAT! G

Default: (disabled)

Option I, Set SabreMixer! Interrupt Number

This option sets the interrupt number that SabreChat!

will expect to find SabreMixer! installed.  The default

for SabreChat! is interrupt $66.  If you suspect an

interrupt conflict with other software, be sure to change

both SabreChat! and SabreMixer! to the same interrupt.

I would suggest placing I$68 on both command lines, this

will change both to interrupt $68. Use this option with

extreme caution and only if you are sure that SabreMixer!

and another TSR are conflicting.

Usage: SCHAT! I<int number>

Default: SCHAT! I$66

Example: SCHAT! I$68

Option K, Automatic Chat Session Capture

By using this option on the command line,  SabreChat!

will automatically capture all chat session into a

file.  The filename created is SCHT!CAP.xxx where xxx

is the conference/room number of the session.

Usage: SCHAT! K

Default: (disabled)

Example: SCHAT! K

Option L, Line-by-line Mode

By putting this option on the command line, SabreChat!

will default to line-by-line mode instead of

character-by-character mode.

Usage  : SCHAT! L

Default: (disabled)

Option M, User Prompting

This command sets the type of prompting that

SabreChat! while use to distinguish between users.

Usage   : SCHAT! M<0..3>

0       No prompting

1       Initial prompting

2       Full name prompting

3       Preceding Fullname

Default : SCHAT! M2

Option N, Select Node [DOOR.SYS]

This command is set by DOOR.SYS.

This option tells SabreChat! which node it is

operating on.  This option is also read from DOOR.SYS

if the Y option is selected and will not need to be

explicitly set.

Usage   : SCHAT! N<node 1..50>

Default : SCHAT! N1 or DOOR.SYS or DORINFO1.DEF

Option O, Read User Information from DORINFO1.DEF.

This option tells SabreChat! to read DORINFO1.DEF for

information pertaining to the user setup.  This

command will automatically set the com port, graphic

mode, and time remaining.  Theses options should not

need to be set once the O command has been used unless

the values are desired to be forced.

Usage   : SCHAT! O<path to DORINFO1.DEF>

Default : (none)

Option P, Select Online Communications Port [DOOR.SYS]

[DORINFO1.DEF]

This command is set by DOOR.SYS and DORINFO1.DEF.

This command sets the communications port (COM) that

it will talk to an online user with.  A port value of

zero is assumed to be a local user.  A communications

fossil will be used to access the port.

Usage   : SCHAT! P<0,1..8> (local,com1..com8)

Default : No ComPort or DOOR.SYS

Option Q, Activate Questionnaire.

This command activates SabreChat! questionnaire

feature.  The files SCHT!QST.TXT and SCHT!ASK.TXT will

need to have been created and SCHT!QST.EXE executed

before this works correctly.

Usage   : SCHAT! Q

Default : (inactive)

Option S, Set SEMAPHORE Path

Remote Access allows you to specify a semaphore path

to a drive for quick access temporary files.  This is

usually a ramdisk drive.  Remote Access will put its

paging files in there.  If you specified RA to use a

semaphore path, then you will need to tell SabreChat!

where this path is with the S<path> command.

Usage   : SCHAT! S<semaphore path>

Default : None

Example : SCHAT! SE:\

Option T, Set User Time Remaining [DOOR.SYS] [DORINFO1.DEF]

This command is set by DOOR.SYS and DORINFO1.DEF.

This option tells SabreChat! how long the user has

access to this door in minutes.  Once this time is

used, the user is returned to the board.

NOTE : This command is ignored in the unregistered

version of SabreChat! and the user time limit

is set to 10 minutes.

Usage   : SCHAT! T<1..255 minutes>

Default : SCHAT! T20 or DOOR.SYS or DORINFO1.DEF.

Option U, Auto-Line Mode Switch

This command line option sets the number of users that

must be in SabreChat! before SabreChat! will

automatically switch to line-by-line mode.  Setting

this value to 0, disables the auto-switch.

Usage   : SCHAT! U<number of users>

Example : SCHAT! U5

-- go to line-by-line mode when the 5th user

enters a SabreChat! conference

Default : SCHAT! U3

Option Y, Read User Information from DOOR.SYS

This option tells SabreChat! to read DOOR.SYS for

information pertaining to the user setup.  This

command will automatically set the user node, com

port, graphic mode, and time remaining.  This options

should not need to be set once the Y command has been

used unless the values are desired to be forced.

Usage   : SCHAT! Y<path to DOOR.SYS>

Default : (none)

Option Z, Set Local Screen Length

This option informs SabreChat! of the size of the

local screen.  Though the size of the local screen is

read from BIOS upon program initialization, you may

desired to force the size if SabreChat! does not read

the value correctly or if you wish to make the

SabreChat! window shorter than screen.  The latter use

is nice if you are using SabreChat! in a windowed

enviroment and you wish to see the status bar at the

bottom of each window.

Usage   : SCHAT! Z<1..50 lines>

Default : AutoDetect

Option &, Immediate User Mode

This option allows you to use SabreChat! without the

need of a BBS user file (NODEINFO.DAT or USERON.BBS).

The user name is set to the name entered on the node

specified by the N option, and on the port specified

by the P option.

Usage   : SCHAT! &<FirstName_LastName>

Default : SCHAT! None

Example : SCHAT! &Shawn_Gresham

Note:  This option also allows you to use SabreChat!

as network chat utility by assigning a node

number to each workstation.  Thus workstation

#5 could just type 'SCHAT! &John_Doe N5' and

JOHN DOE would enter SabreChat! as node 5.

Customizing Menus

�����������������

There are three screens in SabreChat! that can be modified.

These are SCHT!HLP, SCHT!MNU, and SCHT!HI.  Each consists of

two files that end with .ASC, a straight ASCII file, and .ANS,

an ANSI file.  You may modify these files as you wish to give

any extra information or flash to your copy of SabreChat!.

However, modifying the command names in the SCHT!MNU file will

NOT modify the command names in SabreChat!.  This will only

confuse your users.

Also, some ANSI screens may contain commands that SabreChat!

does not correctly interpret.  This screens will appear wrong

on the local side only.

Action Commands

���������������

SabreChat!'s action feature allows the user to make gestures

with just a few keystrokes.  An example would be /SMILE that

sends "SHAWN GRESHAM smiles."  to each user in his conference.

These commands are an attempt to give SabreChat! a more

conversation atmosphere.

The action commands can be altered by modifing the file

SCHT!ACT.TXT.  Before doing so, be sure to change into the

directory containing that file.  Each line of SCHT!ACT.TXT

is in this format:

<COMMAND> <TEXT>

<COMMAND> is a word of ten characters or less that will be used

by the users to invoke this action command.  Verbs usually

work best for this.

<TEXT> is a coded string that is used to produce the action text

that will be displayed to everyone in the chat.

example defined in SCHT!ACT.TXT:

SUCK  There is a vast sucking noise.

example as used in chat:

/SUCK

There is a vast sucking noise.

There are also two special variables that can be used.

These are:

%1      user's name

%2      destination user's name

Example defined in SCHT!ACT.TXT:

JUMP    %1 jumps.

KISS    %1 kisses %2!!

example used in chat:

/JUMP

SHAWN jumps.

/KISS Gina

SHAWN kisses GINA!!

And one last special command that is based off of the sex

of the user.

This is:

@male string@female string@

Example defined in SCHT!ACT.TXT:

FLAP %1 flaps @his@her@ arms.

example used in chat:

/FLAP

SHAWN flags his arms.

SabreChat! will support as many action commands as memory

allows.

After modifying the file SCHT!ACT.TXT, you will need to run

SCHT!ACT.EXE.  This will convert SCHT!ACT.TXT into

SCHT!ACT.DAT that is read by SabreChat!.  SCHT!ACT.EXE will

notify you if you have made any mistakes regarding the size of

an action command.

Questionnaire

�������������

The questionnaire feature of SabreChat! allows you to ask

additional information on a user.  This information will be

available to other users when they encounter each other by

using the /LOOK command.  This feature is an attempt to

'flesh' out a chatting session.  Users will be able to put a

description along with the 'voice' of the person they are

chatting with.

The questionnaire consists of three files: the aspect

definition file (SCHT!QST.TXT), the ask file (SCHT!ASK.TXT),

and the show file (SCHT!SHW.TXT). To set up a questionnaire

file you can edit the files that came in the SabreChat!

archive or by create a new ones by deleting the SCHT!QST.TXT,

SCHT!ASK.TXT, SCHT!SHW.TXT files and doing it from scratch.

For a start, I suggest slightly altering the ones that I made

until you get the hang of it.

Following I will explain how each of these files are set up:

The aspect definition file - SCHT!QST.TXT

-----------------------------------------

We start with the aspect definition file which is the heart of

the questionnaire.  It contains information about the various

aspects of an user.  Each line in this file defines an aspect.

The format for each line is as follows:

<aspect name> <length in characters of entry>

for example:

hair_color      10

defines the aspect HAIR_COLOR that can be up to 10 characters

long.  This information will be used by SabreChat! when the

user answers the questionnaire. Which leads us to...

The ask file - SCHT!ASK.TXT

---------------------------

The ask file is displayed when a user enters SabreChat! for

the first time or when the user wishes to change his/her

current aspects/answers.  Basically, the ASK file asks the

questions that the ASPECT DEFINITION file will contain. The

format for this file is simple.  Everything will be printed as

is except lines that begin with a % (percent) character.  A

line beginning with a percent character is expected to contain

an aspect name (for example HAIR_COLOR).  Using the example

above:

SCHT!ASK.TXT:

What color is your hair?

%HAIR_COLOR%

This sample prints the line 'What color is your hair?' and

then prompts for a string up to 10 characters long as defined

in SCHT!QST.TXT and assigns the string to the name HAIR_COLOR.

The show file - SCHT!SHW.TXT

----------------------------

Where the ASK file asks the questions, the show file shows the

answers.  The show file will appear to the users when they use

the /LOOK command from SabreChat!. The format for this file is

a little more complicated than the previous SCHT!ASK file as

it allows for some minor formatting of the aspects. The text

will printed as is to the user except for the user aspects

which should be prefixed and suffixed with a % (percent)

character.  Using the previous example, you might display the

user's hair color with the following line:

SCHT!SHW.TXT:

Hair color : %HAIR_COLOR%

The show file also allows you to define how big the space is

that the answer will appear in.  Though this may seem awkward

at first, it becomes very powerful when you wish to align your

text into two or more columns.  You can specify the aspect

field size by placing the field size in letters between the

first % (percent) and the aspect name:

%<field size>[ASPECT name]%

Using the example above with a few new aspects:

Hair Color : %38HAIR_COLOR% Eye Color : %EYE_COLOR%

Height     : %38HEIGHT% Weight    : %WEIGHT%

This example would display the four aspects with the WEIGHT

and EYE_COLOR being lined up regardless of the lenght of the

HAIR_COLOR or HEIGHT answers.

The field size is completely optional and is assumed to be the

length of the answer if left out.

Finally

-------

After completing these three files, you will need to run

SCHT!QST.EXE to compile SCHT!QST.TXT into SCHT!QST.DAT.  The

compiler will notify you of any errors found in the TXT file.

And very last, you will need to use the Q option on the

command line to activate the questionnaire.

Technical Information

���������������������

Following is some technical information that may be useful in

debugging application conflicts.

� The communication fossil is expected to be at interrupt 14h.

� NetBIOS is accessed through interrupt 5Ch.

� Interrupt 66h is used to communicate with SabreMixer!.

� NetBIOS datagrams are used in NetBIOS mode.

� Record locking is used to safely access USERON.BBS (ra/tag)

and NODEINFO.DAT (wc) files.

Thanks

������

Thanks to my very very responsive beta testers!!!!  Give these

guys a call to see the lastest SabreChat! in action!

Wildcat BBS

-----------

Greg Day                        Steve Henry

Suburban Computer Connection    AMBASSADOR BOARD

Garden City, MI                 Ontario, Canada

1-313-427-0223                  1-519-925-0724 v.32bis

1:2410/193

Jeff Shapiro                    Chuck Alpert

Computer Junction               Short Circuit

Elizabeth, NJ                   Boca Raton, FL

1-908-354-6979                  1-407-997-2235

1:107/900                       1:369/78

TAG BBS

-------

Allen Crist

Code Three

Prescott, AZ

1-602-771-8646

1:15/23

Remote Access BBS

-----------------

Rick Sande                      Josh Zerlan

LifeStyles BBS                  The Temporal Alliance

Branford, CT                    Ft. Collins, CO

1-203-481-4836                  1-303-490-1047

1:141/107                       1:306/23

Chris DeClerk

Yolanda's Wonderland

State University, AR

1-501-972-6549

1:389/12

Also many thanks to Joel Karr for initially getting me started

in SabreChat! and offering his system as the birth place of

this program.

To Ellis Green of Ellwye's Funhouse the use of his system as

a distribution point and mail contact before my arrival into

the FIDOnet and BBS world.

To Gina Lewter for the many hours of putting up with me and my

'puter and, foremost, for agreeing to be my wife (May 8, 1993!

everybody be there!) Love you babe! (yes, we can go eat now)

And to everyone of you users out there who poke, jab, pummel,

and basically pull the arm of your sysop to register

SabreChat!!!!

Future Versions

���������������

Here are a few ideas that my beta testers and I have come

up with for future versions:

* adding a KNOCK command for users to request entrance to a

private conference (room)

* expanding the IGNORE command to allow a user to ignore a

certain node

* possibly adding a sysop page option and apporiate handling

* detection of users logging on/off the board...

Let me know what you think or if you have any other

suggestions.

Contacting The Author

���������������������

You can reach the author by sending netmail to SHAWN GRESHAM

on the following board:

Maximum Intensity

Whitesboro, Texas, 76273

FIDOnet 1:19/129

1-903-564-3853 22:00 to 06:00 cst

Or by sending a letter to:

Shawn Gresham

Route 1 Box 544

Gordonville, Texas 76245-9789 USA

Or, by dropping a message on the RA_UTIL or WILDCAT echos to

SHAWN GRESHAM.

Any comments or suggestions would be treasured.

If you are having problems and wish to contact me, please

have the following information handy:

BBS System (ra/wildcat/tag)

Machine types. (8086, 80286-10, 80486DX-50, etc)

Memory Manager (if any) (QEMM, 386MAX, etc...)

System Type (DESQview, network, WINDOWS, etc...)

Operating System (MS-DOS 3.3, MS-DOS 5.0, DRDOS 6.0, etc...)

Number of nodes in your system

Command line used for SabreChat! (as in the MAIN1.RUN)

Command line used for SabreMixer! (as in AUTOEXEC.BAT)

Current S!NODE= environment setting (as in AUTOEXEC.BAT)

Current S!EXE= environment setting (as in AUTOEXEC.BAT)

This information is invaluable for debugging SabreChat! problems.

����������������������������������������������

SabreChat! Copyright (C) 1993 Shawn Gresham