Fix_It_To_Run_On_My_System_Doc

Communications are very quirky.  What works fine on one system will bog down

on the next and Not run at all on the next.

I use the Sharkware IGMDrive Ver. 3.08 ( An excellent package ) to create

IGM's.

If you can't get this IGM run on your system try these proven tricks.

If you are running multinode make sure share.exe is installed and it is

better if you set the IGM's .exe file attribute to read_only.

Q. It reads keys from the port but will not write to the port or writes

To the port but will not read from the port.

A.

==========================================================================

If you are running above 9600 USE A FOSSIL!

If your port is Locked and\or you use a fossil driver then in LORDCFG set

Fossil\Internal to Use Extended Fossil or Use Regular Fossil ( Depending

on what works best ).

Set Locked Baud in LORDCFG to your Locked Baud speed. (Usually 38400)

If you are not using a Fossil Driver then set one up to use while in LORD

and the IGM's for it.

I suggest BNU for Dos and SIO for OS/2.  I have been told by several sysop's

that these drivers work well.

If you have some other fossil driver that you prefer, use it instead, as these

are only suggestions not written in stone. <G>

Here is an example of a batch file to call LORD under Dos and load BNU first

then when LORD is done it unloads BNU.

Echo off

cd\LORD_Directory

BNU /L:0=38400 /T:4096 /R:4096 /F+

command to start LORD would go here.

BNU /U

cd\BBS_Directory

Reload BBS

Make sure the fossil driver is in your path.

Change to the LORD Directory

Load the fossil driver.  In the above example the /L:0=38400 designates

comport 1 locked at 38400.  The /T:4096 set the transmit buffer to 4096

bytes.  The /R:4096 sets the recieve buffer to 4096.  The /F+ set it run

FAST ansi.

=============================================================================